- What is computer architecture?
 It is the design and organization of a computer system, including instruction set, CPU, memory, and I/O.

- Difference between computer architecture and organization?
 Architecture is the functional design; organization is the implementation and hardware realization.

- What is ISA (Instruction Set Architecture)?
 ISA defines the set of instructions, data types, registers, and addressing modes for a processor.

- What is microarchitecture?
 Microarchitecture describes how a CPU implements the ISA using registers, ALU, cache, and pipelines.

- Difference between CISC and RISC?
 CISC has complex instructions, fewer registers; RISC has simple instructions, more registers, and pipelining.

- What is a bus in computer architecture?
 A bus is a communication pathway for transferring data between CPU, memory, and I/O devices.

- Difference between address bus, data bus, and control bus?
 Address bus selects memory location, data bus transfers data, control bus carries commands/signals.

- What is a register?
 A small, fast storage inside CPU used to hold instructions, data, or addresses temporarily.

- What is a general-purpose register?
 Registers used for storing intermediate computation results, data, and addresses.

- What is a program counter (PC)?
 PC holds the address of the next instruction to be executed.

- What is ALU?
 Arithmetic Logic Unit performs arithmetic and logical operations.

- What is control unit (CU)?
 CU directs the operation of CPU, controlling instruction fetch, decode, and execution.

- Difference between hardwired and microprogrammed control?
 Hardwired uses fixed logic circuits, microprogrammed uses stored microinstructions.

- What are flags in CPU?
 Special bits indicating the result of arithmetic/logic operations (e.g., zero, carry).

- What is an accumulator?
 A register in CPU used to store intermediate results during arithmetic operations.

- What is instruction register (IR)?
 IR holds the current instruction being decoded/executed.

- What is stack pointer (SP)?
 SP holds the address of the top of the stack in memory.

- What is MAR (Memory Address Register)?
 MAR holds the address in memory for reading or writing data.

- What is MDR (Memory Data Register)?
 MDR holds data being transferred to/from memory.

- What is a status register?
 Contains flags that represent CPU state after an operation.

- What are data transfer instructions?
 Instructions that move data between registers, memory, and I/O (e.g., MOV, LOAD, STORE).

- What are arithmetic instructions?
 Instructions performing addition, subtraction, multiplication, or division.

- What are logical instructions?
 Instructions performing AND, OR, XOR, NOT operations.

- What are control transfer instructions?
 Instructions that alter program flow like jumps, calls, and returns.

- What is an immediate addressing mode?
 Operand is specified directly in the instruction.

- What is register addressing mode?
 Operand is located in a register.

- What is direct addressing mode?
 Instruction contains the memory address of the operand.

- What is indirect addressing mode?
 Instruction specifies a memory location that holds the operand’s address.

- What is relative addressing?
 Operand address is determined relative to PC or base register.

- What is indexed addressing mode?
 Operand address = base address + index register value.

- What is primary memory?
 Fast memory directly accessible by CPU, like RAM and cache.

- What is secondary memory?
 Slower, larger storage like HDD, SSD, and optical disks.

- What is cache memory?
 Small, fast memory between CPU and RAM to reduce access time.

- What are cache levels (L1, L2, L3)?
 L1 is fastest/smallest, L2 larger/slower, L3 largest/slowest among CPU caches.

- Difference between SRAM and DRAM?
 SRAM is fast, volatile, uses flip-flops; DRAM is slower, uses capacitors, needs refresh.

- What is memory latency?
 The delay between memory request and data availability.

- What is memory bandwidth?
 The rate at which data can be read/written to memory.

- What is virtual memory?
 Technique allowing programs to use more memory than physically available using disk.

- What is paging?
 Dividing memory into fixed-size pages for virtual memory management.

- What is segmentation in memory?
 Dividing memory into variable-size segments based on logical program structure.

- What is instruction pipelining?
 Technique of overlapping instruction execution to improve CPU throughput.

- What is pipeline hazard?
 Conditions preventing next instruction from executing in the pipeline.

- What are structural hazards?
 Occur when hardware resources are insufficient to execute instructions simultaneously.

- What are data hazards?
 Occur when an instruction depends on result of previous instruction.

- What are control hazards?
 Occur due to branch instructions affecting instruction flow.

- Difference between in-order and out-of-order execution?
 In-order executes instructions sequentially; out-of-order executes independent instructions early.

- What is branch prediction?
 Technique to guess the outcome of a branch to reduce pipeline stalls.

- What is superscalar architecture?
 CPU can execute multiple instructions per clock cycle using multiple pipelines.

- What is pipeline throughput?
 Number of instructions completed per unit time in a pipeline.

- What is pipeline latency?
 Time required for an instruction to pass through all pipeline stages.

- What is MIPS?
 Million Instructions Per Second, a measure of CPU speed.

- What is CPI?
 Clock Cycles Per Instruction, an average number of cycles per instruction.

- How is CPU execution time calculated?
 Execution time = Instructions × CPI × Clock cycle time.

- What is Amdahl’s Law?
 It predicts maximum speedup using parallelism given the fraction of serial code.

- What is throughput?
 Number of tasks completed per unit time.

- What is latency?
 Time to complete a single task.

- What is speedup?
 Ratio of old execution time to new execution time after improvement.

- What is bottleneck in architecture?
 Component limiting overall system performance.

- What is clock rate?
 Frequency at which a CPU executes instructions.

- What is instruction-level parallelism (ILP)?
 Number of instructions that can be executed simultaneously.

- What is I/O device?
 Hardware for communication between CPU and external environment.

- What is programmed I/O?
 CPU actively waits and controls data transfer with I/O devices.

- What is interrupt-driven I/O?
 CPU is interrupted when I/O device is ready for data transfer.

- What is DMA (Direct Memory Access)?
 Allows I/O devices to transfer data directly to memory without CPU intervention.

- Difference between synchronous and asynchronous I/O?
 Synchronous waits for completion; asynchronous continues execution while I/O occurs.

- What is memory-mapped I/O?
 I/O devices are mapped to memory address space.

- What is isolated I/O?
 Separate address space is used for I/O devices.

- What is polling in I/O?
 CPU repeatedly checks device status to see if ready.

- What is I/O buffer?
 Temporary storage area for data transfer between CPU and I/O devices.

- What is spooling?
 Storing data in a buffer (disk) to be processed by I/O devices later.

- What is R-type instruction?
 Register-to-register operation instruction.

- What is I-type instruction?
 Immediate instruction that uses a constant value.

- What is J-type instruction?
 Jump instruction for changing program flow.

- What is zero-address instruction?
 Operates using stack only, no explicit operands.

- What is one-address instruction?
 Operates with one operand; accumulator used implicitly.

- What is two-address instruction?
 Uses two operands; one also stores the result.

- What is three-address instruction?
 Uses three operands; first operand stores the result.

- What is instruction format?
 Layout of fields (opcode, operands, addressing mode) in instruction.

- What is opcode?
 Part of instruction specifying operation to be performed.

- What is operand?
 Data or address on which instruction operates.

- What is cache hit?
 Requested data found in cache memory.

- What is cache miss?
 Requested data not found in cache, must access main memory.

- What is write-through policy?
 Cache and main memory updated simultaneously.

- What is write-back policy?
 Cache updates memory only when data is replaced.

- What is fully associative cache?
 Any block can go to any cache line.

- What is direct-mapped cache?
 Each block maps to a single cache line.

- What is set-associative cache?
 Cache divided into sets; blocks mapped to any line in a set.

- What is page fault?
 When a program accesses a page not in physical memory.

- What is TLB (Translation Lookaside Buffer)?
 Cache for page table entries to speed up virtual-to-physical address translation.

- What is demand paging?
 Load pages into memory only when required by execution.

- What is superscalar processor?
 CPU that executes multiple instructions per clock cycle using multiple pipelines.

- What is VLIW architecture?
 Very Long Instruction Word; executes multiple operations in one wide instruction.

- What is SIMD?
 Single Instruction, Multiple Data; executes same instruction on multiple data elements.

- What is MIMD?
 Multiple Instruction, Multiple Data; different instructions on different data.

- What is hardware multithreading?
 CPU executes multiple threads simultaneously to improve utilization.

- What is superscalar hazard?
 Pipeline stall due to instruction dependency in multiple pipelines.

- What is speculative execution?
 CPU executes instructions before branch outcome is known to improve speed.

- What is out-of-order completion?
 CPU executes instructions in any order but commits results in program order.