- What is digital logic design?
It is the study of logic gates, circuits, and systems that process digital signals.

- What is the difference between analog and digital signals?
Analog is continuous, digital is discrete in time and amplitude.

- What is a logic gate?
A circuit implementing Boolean functions with binary inputs and outputs.

- What is Boolean algebra?
An algebraic system with values {0,1} and operators AND, OR, NOT.

- State De Morgan’s theorem.
¬(A·B)=¬A+¬B and ¬(A+B)=¬A·¬B.

- What is canonical form?
A standard Boolean expression written as SOP or POS.

- What is SOP form?
Sum of minterms, representing OR of AND terms.

- What is POS form?
Product of maxterms, representing AND of OR terms.

- What is a minterm?
A product term where each variable appears once in complemented or uncomplemented form.

- What is a maxterm?
A sum term where each variable appears once in complemented or uncomplemented form.

- What is K-map?
A graphical tool to simplify Boolean expressions.

- How many cells in a 4-variable K-map?
It has 16 cells.

- What is don’t care condition?
Input combinations that never occur, used to simplify circuits.

- What is combinational circuit?
Output depends only on present input.

- What is sequential circuit?
Output depends on present input and past states.

- What is a half adder?
A circuit adding 2 bits with outputs SUM and CARRY.

- What is a full adder?
A circuit adding 3 bits with outputs SUM and CARRY.

- What is a half subtractor?
A circuit subtracting 2 bits with outputs DIFFERENCE and BORROW.

- What is a full subtractor?
A circuit subtracting 3 bits with outputs DIFFERENCE and BORROW.

- What is a multiplexer?
A device selecting one of many inputs to a single output.

- What is a demultiplexer?
A device sending one input to multiple outputs.

- What is an encoder?
It converts 2^n input lines into n output lines.

- What is a decoder?
It converts n input lines into 2^n output lines.

- What is priority encoder?
An encoder giving priority to highest-order input.

- What is parity bit?
An extra bit added for error detection.

- What is even parity?
The number of 1’s including parity bit is even.

- What is odd parity?
The number of 1’s including parity bit is odd.

- What is a latch?
A level-triggered memory element.

- What is a flip-flop?
An edge-triggered memory element.

- What is SR flip-flop?
A flip-flop with Set and Reset inputs.

- What is D flip-flop?
A flip-flop that stores data from a single input.

- What is JK flip-flop?
A flip-flop that toggles when J=K=1.

- What is T flip-flop?
A flip-flop that toggles on every clock when T=1.

- What is edge triggering?
Change occurs only on rising or falling clock edge.

- What is level triggering?
Change occurs while clock is high or low.

- What is metastability?
An uncertain state of flip-flop due to setup/hold violation.

- What is setup time?
The minimum time input must be stable before clock edge.

- What is hold time?
The minimum time input must remain stable after clock edge.

- What is a counter?
A sequential circuit that counts clock pulses.

- What is ripple counter?
A counter where flip-flops trigger each other sequentially.

- What is synchronous counter?
All flip-flops triggered simultaneously by a common clock.

- What is ring counter?
A shift register with last flip-flop output fed to first.

- What is Johnson counter?
A shift register counter with complemented feedback.

- What is modulus of counter?
The number of unique states the counter cycles through.

- What is a register?
A group of flip-flops storing multiple bits.

- What is shift register?
A register that shifts data left or right.

- What is universal shift register?
It can shift left, shift right, load parallel, and hold data.

- What is asynchronous circuit?
Output changes without global clock synchronization.

- What is synchronous circuit?
Output changes in sync with a global clock.

- What is propagation delay?
The time taken for input change to affect output.

- What is fan-in?
The number of inputs a gate can handle.

- What is fan-out?
The number of gate inputs driven by one gate output.

- What is noise margin?
The tolerance level for input voltage against noise.

- What is power dissipation?
The power consumed by a circuit during operation.

- What is TTL logic family?
Transistor-Transistor Logic, built using bipolar transistors.

- What is CMOS logic family?
Complementary MOSFET-based logic with low power dissipation.

- What is propagation constant in circuits?
It measures signal delay across logic elements.

- What is a PLA?
Programmable Logic Array for implementing custom logic.

- What is a PAL?
Programmable Array Logic with fixed OR gates.

- What is FPGA?
Field Programmable Gate Array for implementing digital systems.

- What is hazard in logic circuits?
An unwanted switching due to different path delays.

- What is static hazard?
Output momentarily changes despite steady inputs.

- What is dynamic hazard?
Output oscillates before settling to correct value.

- What is race condition?
Output depends on the sequence of input changes.

- What is Moore machine?
A finite state machine with output depending only on current state.

- What is Mealy machine?
FSM where output depends on both state and input.

- What is equivalence of Moore and Mealy machines?
Both can represent the same functions with different states.

- What is finite state machine minimization?
Reducing the number of states without changing behavior.

- What is one-hot encoding?
A state machine encoding with one flip-flop per state.

- What is Gray code?
A binary code where consecutive numbers differ by one bit.

- What is BCD code?
Binary Coded Decimal represents each decimal digit with 4 bits.

- What is Hamming code?
An error-detecting and correcting code using parity bits.

- What is the minimum Hamming distance for single error correction?
It must be at least 3.

- What is carry look-ahead adder?
A fast adder that computes carry signals in parallel.

- What is carry save adder?
An adder used in multipliers to avoid carry propagation.

- What is Booth’s algorithm?
An algorithm for fast multiplication using recoding.

- What is restoring division algorithm?
Division method that restores remainder after subtraction.

- What is non-restoring division algorithm?
Division without restoring remainders, faster than restoring.

- What is floating-point representation?
Represents real numbers with mantissa, exponent, and sign.

- What is IEEE 754 standard?
A standard for floating-point arithmetic representation.

- What is normalized floating-point number?
A representation where leading digit is non-zero.

- What is pipeline in digital design?
Breaking a task into stages for parallel execution.

- What is parallelism in circuits?
Executing multiple operations simultaneously to increase speed.

- What is clock skew?
Difference in clock arrival time at different flip-flops.

- What is clock jitter?
Small variations in clock signal timing.

- What is dynamic power consumption in CMOS?
Power due to charging/discharging of capacitances during switching.

- What is static power consumption in CMOS?
Leakage current power when circuit is idle.

- What is setup violation?
When input changes too close to clock edge before latching.

- What is hold violation?
When input changes too early after clock edge.

- What is asynchronous reset?
Reset works independently of the clock.

- What is synchronous reset?
Reset is effective only with the clock edge.

- What is pipeline hazard?
A condition preventing next instruction from executing in pipeline.

- What is structural hazard?
Occurs when hardware resources are insufficient for parallel operations.

- What is data hazard?
Occurs when instruction depends on result of previous instruction.

- What is control hazard?
Occurs due to branch instructions changing program flow.

- What is ROM?
Read-Only Memory used for permanent storage.

- What is PROM?
Programmable ROM that can be programmed once.

- What is EPROM?
Erasable PROM that can be erased by UV light.

- What is EEPROM?
Electrically erasable ROM, reprogrammable multiple times.

- What is SRAM?
Static RAM using flip-flops, faster but expensive.

- What is DRAM?
Dynamic RAM using capacitors, cheaper but needs refreshing.

- What is CAM?
Content Addressable Memory used for associative searching.

- What is microprogramming?
A control technique using microinstructions to implement control logic.

- What is hardwired control?
Control implemented with fixed logic circuits.

- What is difference between RISC and CISC control?
RISC uses simple hardwired control, CISC often uses microprogramming.

- What is instruction pipelining?
Executing multiple instructions in overlapping stages.

- What is instruction-level parallelism?
Executing independent instructions simultaneously.

- What is hazard mitigation?
Techniques like forwarding, stalling, branch prediction.

- What is speculative execution?
Executing instructions before knowing if they’re needed.

- What is branch prediction?
Guessing direction of branch to avoid pipeline stalls.

- What is cache memory?
High-speed memory between CPU and main memory.

- What is direct-mapped cache?
Each block maps to exactly one cache line.

- What is associative cache?
A block can be stored in any cache line.

- What is set-associative cache?
Cache divided into sets, each block maps to one set.

- What is write-through policy?
Data written to cache and main memory simultaneously.

- What is write-back policy?
Data written to memory only when cache line is replaced.

- What is memory hierarchy?
Arrangement of memory types by speed and cost.

- What is virtual memory?
Technique using disk as an extension of RAM.

- What is memory-mapped I/O?
I/O devices accessed using memory addresses.

- What is isolated I/O?
I/O devices accessed with separate I/O instructions.

- What is tri-state buffer?
A buffer with high, low, and high-impedance states.

- What is bus contention?
When multiple devices drive the bus simultaneously.

- What is handshaking in digital systems?
A protocol to synchronize data transfer between devices.

- What is synchronous data transfer?
Transfer occurs under a global clock.

- What is asynchronous data transfer?
Transfer occurs using request/acknowledge signals.

- What is ALU?
Arithmetic Logic Unit performing arithmetic and logic operations.

- What is control unit?
Directs data flow and instruction execution in CPU.

- What is microinstruction format?
Specifies control signals for executing one micro-operation.

- What is PLA-based control unit?
A control unit implemented using programmable logic arrays.

- What is FPGA-based digital design advantage?
Reconfigurable, fast prototyping, and supports parallel execution.