- What is routing?
 Routing is the process of selecting paths for data to travel across networks.

- What is a routing table?
 A routing table stores network paths and the best next-hop for each destination.

- What is a static route?
 A manually configured route that does not change unless updated by an admin.

- What is a dynamic route?
 A route automatically learned and updated by routing protocols.

- Difference between interior and exterior routing protocols?
 Interior operates within an AS (e.g., OSPF, EIGRP), exterior connects ASes (e.g., BGP).

- What is distance vector routing?
 A routing protocol where routers share their routing table with neighbors periodically.

- What is link-state routing?
 Routers maintain a full network map and calculate shortest paths (e.g., OSPF).

- What is hop count?
 The number of routers a packet traverses to reach the destination.

- What is metric in routing?
 A value used to determine the best path, e.g., hop count, bandwidth, delay.

- What is administrative distance?
 A value indicating trustworthiness of a routing source; lower is preferred.

- Difference between RIP v1 and RIP v2?
 v1 is classful; v2 is classless and supports authentication.

- What is split horizon?
 A technique to prevent routing loops by not advertising routes back to the source.

- What is route poisoning?
 Setting a route metric to infinity to indicate the network is unreachable.

- What is triggered update?
 Immediate update sent when a network change occurs, instead of waiting for timer.

- What is OSPF?
 Open Shortest Path First is a link-state IGP using SPF algorithm.

- What is OSPF area?
 A logical grouping of OSPF networks to reduce routing overhead.

- What is OSPF backbone area?
 Area 0; all other OSPF areas must connect to it.

- What is OSPF router ID?
 A unique 32-bit identifier for each OSPF router.

- Difference between OSPF DR and BDR?
 DR is the Designated Router; BDR is the Backup Designated Router for redundancy.

- What is OSPF cost?
 A metric based on interface bandwidth; lower cost is preferred.

- What is EIGRP?
 Enhanced Interior Gateway Routing Protocol, a hybrid routing protocol using DUAL.

- What is DUAL algorithm?
 Diffusing Update Algorithm calculates loop-free paths in EIGRP.

- Difference between EIGRP metric and OSPF metric?
 EIGRP uses bandwidth, delay, load, reliability; OSPF uses cost based on bandwidth.

- What is BGP?
 Border Gateway Protocol, an exterior gateway protocol for inter-AS routing.

- What is AS in BGP?
 Autonomous System; a collection of networks under a common routing policy.

- Difference between iBGP and eBGP?
 iBGP is within AS, eBGP connects different ASes.

- What is BGP path vector?
 Keeps track of AS path to avoid loops.

- What is route summarization?
 Combining multiple subnets into a single route to reduce routing table size.

- What is default route?
 Route used when no specific path exists for a destination.

- Difference between classful and classless routing?
 Classful ignores subnet mask, classless considers it.

- What is route redistribution?
 Sharing routes between different routing protocols.

- What is convergence?
 Time taken by routers to reach consistent routing tables after a change.

- What is routing loop?
 Continuous circulation of a packet due to incorrect routing information.

- Difference between interior and exterior BGP?
 iBGP operates inside AS; eBGP connects ASes.

- What is next-hop in routing table?
 The IP address of the next router to forward a packet.

- What is administrative distance of OSPF?
 110.

- What is administrative distance of EIGRP?
 90 for internal routes.

- What is route flapping?
 Frequent changes in network reachability causing instability.

- What is passive interface in routing?
 Prevents sending routing updates on an interface but still receives updates.

- What is multicast routing?
 Routing packets to multiple destinations using group addresses.

- Difference between RIPv1 and RIPv2?
 v1 is classful, v2 supports classless, multicast, and authentication.

- What is VRF?
 Virtual Routing and Forwarding; creates multiple routing tables on one router.

- What is policy-based routing?
 Routing packets based on policies, not just destination IP.

- Difference between unicast, multicast, and broadcast routing?
 Unicast to one, multicast to group, broadcast to all nodes.

- What is OSPF hello protocol?
 Sends hello packets to discover neighbors and maintain adjacency.

- What is EIGRP neighbor table?
 Stores information about directly connected EIGRP routers.

- Difference between OSPF and IS-IS?
 Both are link-state, but IS-IS uses CLNS addressing, mostly ISP networks.

- What is BGP route reflectors?
 Reduces iBGP full mesh requirement by reflecting routes to other BGP routers.

- What is BGP attributes?
 Metrics like AS-path, NEXT_HOP, LOCAL_PREF, used for path selection.

- What is BGP weight attribute?
 Cisco-specific local preference for path selection; higher weight is preferred.

- What is a VLAN?
 Virtual LAN; separates devices logically on the same physical network.

- What is VLAN trunking?
 Allows multiple VLANs to pass through a single link using tags.

- What is 802.1Q?
 VLAN tagging standard used in trunk links.

- Difference between access port and trunk port?
 Access carries one VLAN; trunk carries multiple VLANs.

- What is native VLAN?
 VLAN on a trunk link that carries untagged frames.

- What is inter-VLAN routing?
 Routing traffic between VLANs using a Layer 3 device.

- What is VTP?
 VLAN Trunking Protocol; shares VLAN info across switches.

- Difference between VTP server, client, and transparent mode?
 Server creates/deletes VLANs, client receives updates, transparent forwards but doesn’t sync.

- What is STP?
 Spanning Tree Protocol prevents switching loops in Layer 2 networks.

- What is RSTP?
 Rapid Spanning Tree Protocol; faster convergence than STP.

- What is MSTP?
 Multiple Spanning Tree Protocol; supports multiple spanning trees per VLAN group.

- What is BPDU?
 Bridge Protocol Data Unit; used to exchange STP info.

- What is root bridge?
 The switch with the lowest bridge ID elected by STP as root.

- What is port role in STP?
 Root, designated, and blocked determine forwarding and blocking state.

- Difference between blocking, listening, learning, and forwarding?
 STP port states: blocking=no traffic, listening=STP negotiation, learning=learn MAC, forwarding=forward traffic.

- What is EtherChannel?
 Combines multiple physical links into one logical link for redundancy and bandwidth.

- Difference between PAgP and LACP?
 PAgP is Cisco proprietary; LACP is IEEE standard for link aggregation.

- What is MAC address table?
 Switch table mapping MAC addresses to physical ports.

- What is difference between store-and-forward and cut-through switching?
 Store-and-forward checks CRC, cut-through forwards immediately.

- What is VLAN pruning?
 Removing unnecessary VLANs from a trunk link.

- What is PVST?
 Per VLAN Spanning Tree; runs separate STP instance per VLAN.

- What is VLAN hopping attack?
 Unauthorized access to other VLANs via double tagging or switch spoofing.

- What is PortFast?
 STP feature to skip listening/learning on edge ports.

- What is BPDU Guard?
 Shuts down port receiving BPDUs unexpectedly (for PortFast ports).

- What is root guard?
 Prevents switches from becoming root bridge unexpectedly.

- What is loop guard?
 Prevents alternate/backup ports from becoming designated during unidirectional link failure.

- Difference between hub and switch?
 Hub broadcasts to all ports; switch forwards based on MAC address.

- What is collision domain?
 Network segment where packets can collide; switches reduce collision domains.

- What is broadcast domain?
 Network segment where broadcast frames are forwarded; VLANs separate broadcast domains.

- What is difference between Layer 2 and Layer 3 switch?
 Layer 2 forwards frames, Layer 3 routes packets between VLANs.

- What is EIGRP neighbor discovery?
 Routers exchange hello packets to establish neighbors.

- What is EIGRP topology table?
 Stores all learned routes and feasible successors.

- What is feasible distance in EIGRP?
 Lowest calculated metric to reach a destination.

- What is reported distance in EIGRP?
 Metric reported by neighbor for a route.

- What is OSPF hello interval?
 Time between hello packets; default 10s on LAN.

- What is OSPF dead interval?
 Time router waits without receiving hello before declaring neighbor down; default 40s.

- What is OSPF cost calculation?
 Cost = 100 Mbps / interface bandwidth.

- What is OSPF LSA?
 Link State Advertisement; describes a router’s links to neighbors.

- What is LSA type 1–5?
 Type 1=router, 2=network, 3=summary, 4=ASBR, 5=external.

- What is OSPF adjacency?
 State where two routers exchange LSAs for routing info.

- What is EIGRP split-horizon?
 Prevents routing info from being sent back on the receiving interface.

- Difference between OSPF and EIGRP convergence?
 EIGRP converges faster due to DUAL algorithm; OSPF uses SPF calculation.

- What is BGP route aggregation?
 Combining multiple prefixes into a summarized route advertisement.

- What is BGP community attribute?
 Tags routes for applying routing policies.

- What is BGP next-hop?
 IP address used to reach a BGP route.

- What is iBGP full mesh requirement?
 iBGP routers must connect to all other iBGP routers to avoid loops.

- What is BGP local preference?
 Attribute for selecting outbound path within an AS; higher preferred.

- What is BGP MED?
 Multi-Exit Discriminator; suggests preferred inbound path to external AS.

- Difference between static and default route?
 Static is manually configured to a specific network; default (*) is catch-all.

- What is passive-interface in OSPF?
 Prevents OSPF from sending updates on the interface.

- What is difference between RIP and OSPF metric?
 RIP uses hop count; OSPF uses bandwidth-based cost.

- What is route summarization in OSPF?
 Aggregating subnets into one advertisement to reduce routing table.

- What is OSPF backbone requirement?
 All areas must connect to Area 0.

- What is EIGRP variance?
 Determines which unequal-cost routes are feasible for load balancing.

- Difference between EIGRP internal and external routes?
 Internal routes learned within AS, external routes redistributed from other protocols.

- What is EIGRP stub routing?
 Prevents EIGRP queries from propagating to stub routers.

- Difference between OSPF NSSA and stub area?
 Stub blocks external LSAs; NSSA allows limited external routes.

- What is BGP AS-path?
 List of ASes a route traversed; helps prevent loops.

- Difference between OSPF and IS-IS hello packets?
 Both discover neighbors; IS-IS uses CLNS frames, OSPF uses IP.

- What is BGP route flap damping?
 Suppresses unstable routes to prevent network instability.

- What is HSRP?
 Hot Standby Router Protocol; provides default gateway redundancy.

- Difference between HSRP and VRRP?
 HSRP is Cisco proprietary; VRRP is open standard.

- What is GLBP?
 Gateway Load Balancing Protocol; provides multiple active gateways.

- What is EtherChannel load balancing?
 Distributes traffic over member links using MAC, IP, or port hash.

- Difference between active and passive LACP?
 Active initiates aggregation; passive responds to negotiation.

- What is port-security?
 Restricts MAC addresses on a switch port for security.

- What is sticky MAC?
 Switch learns MAC dynamically and adds it to running config.

- What is PVST+?
 Per VLAN Spanning Tree Plus; supports VLAN-specific STP on Cisco switches.

- What is MST region?
 Set of switches configured to map multiple VLANs to fewer spanning trees.

- Difference between PVST+ and MST?
 PVST+ runs one STP per VLAN; MST maps multiple VLANs to one STP instance.

- What is RSTP rapid convergence?
 Uses edge ports and alternate ports to converge in seconds.

- What is DHCP snooping?
 Security feature that filters untrusted DHCP messages.

- What is ARP inspection?
 Prevents ARP spoofing by validating ARP packets.

- What is storm control?
 Limits broadcast, multicast, or unicast traffic to prevent network overload.

- What is MPLS?
 Multiprotocol Label Switching; routes packets using labels instead of IP.

- What is LDP in MPLS?
 Label Distribution Protocol; distributes labels in MPLS networks.

- What is QoS?
 Quality of Service; prioritizes traffic for performance.

- What is difference between DiffServ and IntServ?
 DiffServ marks traffic for class-based QoS; IntServ reserves resources per flow.

- What is ACL?
 Access Control List; filters traffic based on IP, protocol, or port.

- Difference between standard and extended ACL?
 Standard filters by source IP; extended filters by source, dest, protocol, port.

- What is NAT?
 Network Address Translation; converts private IP to public IP.

- What is PAT?
 Port Address Translation; multiple private IPs share one public IP.

- Difference between static and dynamic NAT?
 Static maps one-to-one, dynamic maps many-to-many using a pool.

- What is DMZ?
 Demilitarized Zone; network segment exposed to untrusted network.

- Difference between hub-and-spoke and full-mesh topology?
 Hub-and-spoke uses a central router; full-mesh connects all nodes directly.

- What is IP SLA?
 Measures network performance like latency, jitter, and packet loss.

- What is OSPF virtual link?
 Connects an area to backbone when physically disconnected.

- Difference between unicast, multicast, broadcast routing?
 Unicast=one-to-one, multicast=one-to-many, broadcast=one-to-all.

- What is switch stacking?
 Combines multiple switches to operate as one logical switch.

- What is VSS?
 Virtual Switching System; two switches act as single logical switch for redundancy.

- Difference between VSS and StackWise?
 VSS merges control plane; StackWise merges data/control plane physically.

- What is SDN?
 Software-Defined Networking; separates control plane from data plane.

- Difference between centralized and distributed control plane?
 Centralized uses one controller (SDN); distributed runs control on each device.

- What is VXLAN?
 Virtual Extensible LAN; overlays Layer 2 on Layer 3 network.

- What is difference between MPLS and VLAN?
 MPLS routes using labels across L3; VLAN segments L2 networks.

- What is VTP pruning?
 Reduces unnecessary VLAN traffic on trunk links.

- What is GRE tunnel?
 Encapsulates packets for point-to-point tunneling over IP networks.

- Difference between IPsec and GRE?
 IPsec provides security; GRE only encapsulates packets.

- What is NAT overload?
 Many internal hosts share one public IP with different ports.

- What is switch virtual interface (SVI)?
 Logical interface for VLAN to route traffic on Layer 3 switch.

- What is PIM sparse mode?
 Protocol Independent Multicast; sends multicast only when receivers join.

- Difference between PIM sparse and dense mode?
 Sparse uses explicit join, dense floods then prunes.

- What is IGMP?
 Internet Group Management Protocol; manages multicast group membership.

- What is difference between IGMP v1, v2, v3?
 v3 supports source-specific multicast.

- What is multicast routing table?
 Stores multicast groups and outgoing interfaces.

- What is MAC flooding attack?
 Overloads switch MAC table to force flooding.

- What is DHCP snooping binding table?
 Stores trusted IP-MAC-port mappings for security.

- Difference between routed port and switchport?
 Routed port functions like L3 interface; switchport is L2.

- What is SPAN?
 Switched Port Analyzer; mirrors traffic for monitoring.

- What is RSPAN?
 Remote SPAN; mirrors traffic across switches.

- What is L3 EtherChannel?
 Aggregates multiple L3 interfaces into one logical interface.

- Difference between access and trunk VLAN on EtherChannel?
 Access carries single VLAN; trunk carries multiple VLANs.

- What is VRF-lite?
 Virtual Routing and Forwarding on a router without MPLS.

- What is redundant links in switching?
 Multiple links for redundancy; STP prevents loops.

- What is frame flooding?
 Switch sends unknown destination frame to all ports.

- What is switch port mode dynamic desirable?
 Negotiates to become trunk using DTP actively.

- What is switch port mode dynamic auto?
 Negotiates to become trunk only if neighbor initiates.

- Difference between Layer 2 and Layer 3 VLAN?
 L2 VLAN segments switchports; L3 VLAN has SVI for routing.

- What is multicast VLAN registration (MVR)?
 Reduces multicast traffic to VLAN members only.

- Difference between static and dynamic routing?
 Static manually configured; dynamic uses protocol updates.

- What is EIGRP unequal-cost load balancing?
 Allows multiple paths with different metrics using variance.

- What is OSPF virtual link authentication?
 MD5 or plaintext authentication between backbone and non-backbone areas.

- Difference between OSPF area types: backbone, stub, NSSA?
 Backbone=Area0, stub=no external LSAs, NSSA=limited external LSAs.

- What is EIGRP stub keyword “receive-only”?
 Prevents router from sending queries, only receives updates.

- What is floating static route?
 Backup static route with higher administrative distance.

- Difference between OSPF full and DR adjacency?
 Full=full LSA exchange; DR adjacency only with root switch on multiaccess LAN.

- What is route tagging?
 Marks redistributed routes for filtering or policy routing.

- What is OSPF backbone summarization?
 Aggregates networks in ABR to reduce LSAs.

- Difference between EIGRP internal and external route AD?
 Internal=90, external=170.

- What is BGP next-hop-self?
 Sets next-hop to router itself when advertising iBGP routes.

- What is OSPF LSA flooding?
 Propagates LSAs to all routers in area for network topology info.

- Difference between BGP local-AS and remote-AS?
 Local-AS=AS of sender, remote-AS=neighbor’s AS.

- What is switch MAC aging?
 Removes inactive MAC entries after timer expires.

- Difference between sticky and static MAC address?
 Sticky learned dynamically and saved; static manually configured.

- What is broadcast storm?
 Excessive broadcast traffic causing network congestion.

- Difference between hub, switch, router?
 Hub=L1, switch=L2, router=L3 device for forwarding.

- What is VLAN access-map?
 ACL applied per VLAN for security.

- Difference between OSPF LSA types 3 & 5?
 Type 3=inter-area summary, Type 5=external routes.

- What is EIGRP variance?
 Determines unequal-cost paths for load balancing.

- Difference between single-area and multi-area OSPF?
 Multi-area reduces SPF calculation and LSAs.

- What is switchback redundancy?
 Backup switch link or device for high availability.

- Difference between routed port and SVI?
 Routed port=L3 interface, SVI=VLAN interface on switch.

- What is multicast routing table?
 Shows group addresses and outgoing interfaces.

- Difference between static NAT and PAT?
 Static maps one-to-one; PAT maps many-to-one with port translation.

- What is VRRP priority?
 Determines which router becomes master; higher wins.

- What is switch VLAN membership?
 Assigns ports to VLANs for segmentation.

- Difference between L2 and L3 STP?
 L2 runs per VLAN; L3 STP integrates routing for loop avoidance.

- What is redundant HSRP group?
 Multiple routers providing backup default gateway for failover.