IPv6 Protocol Engineering & Deployment Assignment

Complete Student Submission Report & Technical Solution Manual

| Student Name: [Your Name] | Course: CSE - Computer Networks |
| --- | --- |
| Student ID: [Your Student ID] | Submission Date: September 2026 |

Part 1: Theoretical & Architectural Analysis

1.1 Address Architecture & Shortening Rules
IPv6 expands the address space to 128 bits (340 undecillion addresses), written in hexadecimal grouped into eight 16-bit blocks (hextets). Two standard compression rules apply:
• Leading Zero Compression: Leading zeros within any 16-bit field are omitted (e.g., 0db8 becomes db8).
• Zero Block Compression (::): Contiguous sequences of all-zero fields are replaced with a double colon (::). This can only be applied once per address to avoid ambiguity.

1.2 Core IPv6 Address Types Summary

| Address Type | Prefix | Scope | Primary Use Case |
| --- | --- | --- | --- |
| Global Unicast (GUA) | 2000::/3 | Global / Internet | Publicly routable communication |
| Unique Local (ULA) | fd00::/8 | Internal Network | Private organizational routing |
| Link-Local (LLA) | fe80::/10 | Single Data Link | NDP, SLAAC, next-hop routing |
| Multicast | ff00::/8 | Defined by Scope | One-to-many group transmission |
| Anycast | Allocated from GUA | Nearest Interface | DNS root servers, load balancing |

1.3 Address Configuration: SLAAC vs. Stateful vs. Stateless DHCPv6
• SLAAC (Stateless Address Autoconfiguration): Hosts listen to ICMPv6 Router Advertisements (RA) to obtain the /64 prefix and auto-generate the 64-bit Interface Identifier using EUI-64 or random privacy extensions.
• Stateless DHCPv6: Hosts obtain IPv6 addresses and default gateway from SLAAC (RA messages) but query a DHCPv6 server for additional parameters such as DNS and domain search lists.
• Stateful DHCPv6: A dedicated DHCPv6 server allocates and tracks IPv6 addresses and leases, identical to IPv4 DHCP operation.

Part 2: Practical Lab Solutions & Implementations

Lab Task A: Linux (Ubuntu) IPv6 Configuration & Verification
To configure a static Global Unicast Address and verify neighbor connectivity:

# 1. Assign Static IPv6 Address to eth0 sudo ip -6 addr add 2001:db8::1/64 dev eth0 # 2. Add Default IPv6 Gateway sudo ip -6 route add default via fe80::1 dev eth0 # 3. Verify Interface Assignment ip -6 addr show dev eth0 # 4. Test Connectivity & Route Tracing ping6 -c 4 2001:db8::2 traceroute6 2001:db8::2 ip -6 neigh show # Verify Neighbor Discovery cache (replaces ARP)

Lab Task B: Linux DHCPv6 Server Configuration (ISC-DHCP-Server)
Configuration file setup for /etc/dhcp/dhcpd6.conf:

# /etc/dhcp/dhcpd6.conf default-lease-time 2592000; preferred-lifetime 604800; option dhcp-renewal-time 3600; option dhcp-rebinding-time 7200; # Subnet declaration subnet6 2001:db8:1:2::/64 { range6 2001:db8:1:2::100 2001:db8:1:2::200; option dhcp6.name-servers 2001:db8:1:2::1; option dhcp6.domain-search "engineering.corp.local"; } # Service Management & Firewall Commands # sudo systemctl restart isc-dhcp-server6 # sudo ufw allow 547/udp comment 'Allow DHCPv6 Server Port'

Lab Task C: Departmental Subnetting Design
Scenario: Allocate assigned prefix 2001:db8:1::/48 across 3 departments (HR, Engineering, Finance), each requiring 10 subnets (minimum 30 subnets total, standard /64):
• Prefix Structure: A /48 allocation provides 16 subnetting bits (bits 48 to 64), yielding 2^16 = 65,536 available /64 subnets (0000 to ffff).
• Department Subnet Plan Table:

| Department | Subnet ID Range | Prefix Allocated | Example Subnet |
| --- | --- | --- | --- |
| Human Resources (HR) | Subnets 0001 to 000a (10 subnets) | 2001:db8:1:0000::/52 | 2001:db8:1:1::/64 to 2001:db8:1:a::/64 |
| Engineering (ENG) | Subnets 0010 to 001a (10 subnets) | 2001:db8:1:0010::/52 | 2001:db8:1:10::/64 to 2001:db8:1:1a::/64 |
| Finance (FIN) | Subnets 0020 to 002a (10 subnets) | 2001:db8:1:0020::/52 | 2001:db8:1:20::/64 to 2001:db8:1:2a::/64 |

Lab Task D: AWS Cloud IPv6 Deployment Steps
1. VPC Creation: Create an Amazon VPC and select 'Amazon-provided IPv6 CIDR block' to obtain a public /56 Global Unicast block.
2. Subnet Allocation: Create public and private subnets, assigning a /64 prefix to each from the allocated /56 block.
3. Route Tables & Gateway: Attach an Internet Gateway (IGW) for public subnets and an Egress-Only Internet Gateway (EIGW) for private subnets (enabling outbound IPv6 without allowing unsolicited inbound connections).
4. Security Group Rules: Authorize inbound ICMPv6 and HTTPS traffic (`::/0`).

Part 3: Troubleshooting Case Study — SLAAC RA Flag Diagnostics

Case Problem: Clients fail to auto-configure IPv6 addresses via SLAAC or fail to receive DHCPv6 parameters.
Root Cause Analysis (Router Advertisement Flags in radvd / Cisco IOS):
1. Autonomous Flag (A-flag): When set to 0, hosts are prohibited from creating SLAAC addresses using the advertised prefix. Fix: Set AdvAutonomous on in radvd.conf.
2. Managed Address Configuration Flag (M-flag): When set to 1, instructs hosts to obtain IP addresses directly from a Stateful DHCPv6 server. When 0, SLAAC is used.
3. Other Configuration Flag (O-flag): When set to 1, instructs hosts to query Stateless DHCPv6 for DNS/NTP options while keeping their SLAAC address.
Cisco IOS Router Configuration Fix:

! Cisco Router Interface Fix for SLAAC + Stateless DHCPv6 interface GigabitEthernet0/0 ipv6 address 2001:db8:1:1::1/64 no ipv6 nd managed-config-flag ! M-flag = 0 (Use SLAAC for address) ipv6 nd other-config-flag ! O-flag = 1 (Query DHCPv6 for DNS) ipv6 nd prefix 2001:db8:1:1::/64 no-autoconfig ! Remove to enable A-flag no shutdown