Efficient Mitchell’s Approximate Log Multipliers for Convolutional Neural Networks

Min Soo Kim, Alberto A. Del Barrio, Leonardo Tavares Oliveira, Roman Hermida, ´ Senior Member, IEEE, Nader Bagherzadeh, Fellow, IEEE.

Describe the working principle of the proposed approximate logarithmic multiplier and explain how it achieves energy efficiency in CNN applications.

Answer: The proposed approximate logarithmic multiplier is based on Mitchell’s logarithmic multiplication algorithm. It converts multiplication into logarithm addition, which reduces the hardware complexity compared with conventional multipliers. To further reduce hardware cost, the design uses w-truncation, improved Leading-One Detector (LOD), optimized shift calculations, and simplified decoder and zero-detection units. These techniques reduce the number of hardware operations and consequently lower power and energy consumption. The multiplier is mainly applied during CNN inference, where approximate computation can provide significant resource savings with only a small accuracy degradation.

For example, at w=8, the proposed multiplier achieves about 88% less energy consumption than an exact 32-bit fixed-point multiplier, with only 0.2% performance degradation on the ILSVRC2012 validation dataset.

Therefore, the proposed multiplier achieves energy-efficient CNN inference by trading a small amount of computational accuracy for significant hardware and energy savings.

প্রস্তাবিত Approximate Logarithmic Multiplier এর কার্যপ্রণালী বর্ণনা কর এবং CNN applications এ এটি কীভাবে energy efficiency অর্জন করে তা ব্যাখ্যা কর।

প্রস্তাবিত Approximate Logarithmic Multiplier মূলত Mitchell’s Logarithmic Multiplication Algorithm এর উপর ভিত্তি করে তৈরি। এখানে সরাসরি multiplication করার পরিবর্তে multiplication কে logarithm এর addition এ রূপান্তর করা হয়। এর ফলে প্রচলিত multiplier এর তুলনায় hardware complexity কমে যায়।

Hardware cost আরও কমানোর জন্য এই design এ w-truncation, উন্নত Leading One Detector (LOD), optimized shift calculation এবং simplified decoder ও zero detection unit ব্যবহার করা হয়েছে। এর মাধ্যমে hardware operations এর সংখ্যা কমে যায় এবং ফলে power ও energy consumption কমে।

এই multiplier মূলত CNN inference এর সময় ব্যবহার করা হয়। CNN inference এ approximate computation ব্যবহার করলে সামান্য performance loss এর বিনিময়ে অনেক hardware resource ও energy সাশ্রয় করা সম্ভব।

উদাহরণস্বরূপ, w=8 হলে proposed multiplier একটি exact 32-bit fixed-point multiplier এর তুলনায় প্রায় 88% কম energy ব্যবহার করে। এর বিনিময়ে ILSVRC2012 validation dataset এ মাত্র 0.2% performance degradation দেখা যায়।