Packet Tracer Skills Integration: Solution & Lab Report

Ch1 - Introduction to Routing and Packet Forwarding (VLSM & Configuration Guide)

Task 1: Design and Document an Addressing Scheme (VLSM)

Base Network Space: 192.168.1.0/24
Subnetting Methodology (Highest Host Requirement First):
1. B1 LAN (80 Hosts required): Requires 7 host bits (2^7 - 2 = 126 hosts). Subnet mask is /25 (255.255.255.128).
   • Network: 192.168.1.0/25
   • Usable Range: 192.168.1.1 to 192.168.1.126 (First IP to B1 Fa0/0, Last IP to PC1).
2. HQ LAN (40 Hosts required): Requires 6 host bits (2^6 - 2 = 62 hosts). Subnet mask is /26 (255.255.255.192).
   • Network: 192.168.1.128/26
   • Usable Range: 192.168.1.129 to 192.168.1.190 (First IP to HQ Fa0/0, Last IP to PC2).
3. B2 LAN (20 Hosts required): Requires 5 host bits (2^5 - 2 = 30 hosts). Subnet mask is /27 (255.255.255.224).
   • Network: 192.168.1.192/27
   • Usable Range: 192.168.1.193 to 192.168.1.222 (First IP to B2 Fa0/0, Last IP to PC3).
4. WAN Link: B1 to HQ (2 Hosts required): Requires 2 host bits (2^2 - 2 = 2 hosts). Subnet mask is /30 (255.255.255.252).
   • Network: 192.168.1.224/30
   • Usable Range: 192.168.1.225 (HQ S0/0/0) and 192.168.1.226 (B1 S0/0/0).
5. WAN Link: HQ to B2 (2 Hosts required): Requires 2 host bits (2^2 - 2 = 2 hosts). Subnet mask is /30 (255.255.255.252).
   • Network: 192.168.1.228/30
   • Usable Range: 192.168.1.229 (HQ S0/0/1) and 192.168.1.230 (B2 S0/0/1).

Complete Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | Fa0/0 | 192.168.1.129 | 255.255.255.192 | N/A |
| HQ | S0/0/0 | 192.168.1.225 | 255.255.255.252 | N/A |
| HQ | S0/0/1 (DCE) | 192.168.1.229 | 255.255.255.252 | N/A |
| B1 | Fa0/0 | 192.168.1.1 | 255.255.255.128 | N/A |
| B1 | S0/0/0 (DCE) | 192.168.1.226 | 255.255.255.252 | N/A |
| B2 | Fa0/0 | 192.168.1.193 | 255.255.255.224 | N/A |
| B2 | S0/0/1 | 192.168.1.230 | 255.255.255.252 | N/A |
| PC1 | NIC | 192.168.1.126 | 255.255.255.128 | 192.168.1.1 |
| PC2 | NIC | 192.168.1.190 | 255.255.255.192 | 192.168.1.129 |
| PC3 | NIC | 192.168.1.222 | 255.255.255.224 | 192.168.1.193 |

Task 3: Router & Host CLI Configuration Scripts

Router HQ Configuration:

Router> enable Router# configure terminal Router(config)# hostname HQ HQ(config)# enable secret class HQ(config)# line console 0 HQ(config-line)# password cisco HQ(config-line)# login HQ(config-line)# exit HQ(config)# line vty 0 4 HQ(config-line)# password cisco HQ(config-line)# login HQ(config-line)# exit HQ(config)# interface FastEthernet 0/0 HQ(config-if)# ip address 192.168.1.129 255.255.255.192 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/0/0 HQ(config-if)# ip address 192.168.1.225 255.255.255.252 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/0/1 HQ(config-if)# ip address 192.168.1.229 255.255.255.252 HQ(config-if)# clock rate 64000 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# end HQ# copy running-config startup-config

Router B1 Configuration:

Router# configure terminal Router(config)# hostname B1 B1(config)# enable secret class B1(config)# line console 0 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# line vty 0 4 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# interface FastEthernet 0/0 B1(config-if)# ip address 192.168.1.1 255.255.255.128 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface Serial 0/0/0 B1(config-if)# ip address 192.168.1.226 255.255.255.252 B1(config-if)# clock rate 64000 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# end B1# copy running-config startup-config

Router B2 Configuration:

Router# configure terminal Router(config)# hostname B2 B2(config)# enable secret class B2(config)# line console 0 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# line vty 0 4 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# interface FastEthernet 0/0 B2(config-if)# ip address 192.168.1.193 255.255.255.224 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface Serial 0/0/1 B2(config-if)# ip address 192.168.1.230 255.255.255.252 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# end B2# copy running-config startup-config

Task 4: Connectivity Test Results

• Can PC1 ping PC2? Yes (Successful, 100%)
• Can PC1 ping PC3? Yes (Successful, 100%)
• Can PC3 ping PC2? Yes (Successful, 100%)

Task 5: Layer 2 and Layer 3 Address Tracking (Ping: PC1 to PC3)

Simulation tracking of an ICMP echo request packet initiated from PC1 (192.168.1.126) destined for PC3 (192.168.1.222):

Step 2: Addresses at PC1 (Outgoing to B1 Fa0/0)
• Layer 3 Source: 192.168.1.126 (PC1 IP)
• Layer 3 Destination: 192.168.1.222 (PC3 IP)
• Layer 2 Source: MAC Address of PC1 NIC
• Layer 2 Destination: MAC Address of B1 Fa0/0 Interface (Default Gateway)

Step 3: Addresses at B1 (Forwarding out S0/0/0 to HQ)
• Layer 3 Source: 192.168.1.126 (Unchanged)
• Layer 3 Destination: 192.168.1.222 (Unchanged)
• Layer 2 Source: HDLC / PPP / Serial Broadcast (No MAC address on point-to-point serial links)
• Layer 2 Destination: HDLC / PPP / Serial Broadcast (0x8F / 0xFF)

Step 4: Addresses at HQ (Forwarding out S0/0/1 to B2)
• Layer 3 Source: 192.168.1.126 (Unchanged)
• Layer 3 Destination: 192.168.1.222 (Unchanged)
• Layer 2 Source: HDLC / PPP / Serial Broadcast
• Layer 2 Destination: HDLC / PPP / Serial Broadcast

Step 5: Addresses at B2 (Forwarding out Fa0/0 to PC3)
• Layer 3 Source: 192.168.1.126 (Unchanged)
• Layer 3 Destination: 192.168.1.222 (Unchanged)
• Layer 2 Source: MAC Address of B2 Fa0/0 Interface
• Layer 2 Destination: MAC Address of PC3 NIC