Packet Tracer Skills Integration Challenge: Solution & Lab Report

Ch2 - Static Routing & Summarization (Packet Tracer 2.9.1 Implementation Guide)

Task 1: Topology & Cabling Requirements

Physical Cabling Setup:
• HQ Serial 0/0/0 (DCE) connects to B1 Serial 0/0/0 (DTE) using a Serial DCE cable.
• HQ Serial 0/0/1 (DCE) connects to B2 Serial 0/0/0 (DTE) using a Serial DCE cable.
• HQ acts as the DCE clock provider on both WAN links (clock rate 64000).
• LAN connections utilize straight-through FastEthernet cables connecting switch ports to router FastEthernet interfaces and client PCs.

Task 2: Complete Addressing Table

Standard Standardized Addressing Scheme:
• HQ LANs: 172.16.1.0/24 (Fa0/0) and 172.16.2.0/24 (Fa0/1).
• B1 LANs (4 Subnets): 172.16.16.0/24 (Fa0/0), 172.16.17.0/24 (Fa0/1), 172.16.18.0/24 (Fa1/0), 172.16.19.0/24 (Fa1/1).
• B2 LANs (4 Subnets): 172.16.32.0/24 (Fa0/0), 172.16.33.0/24 (Fa0/1), 172.16.34.0/24 (Fa1/0), 172.16.35.0/24 (Fa1/1).
• WAN Links: HQ to B1 = 172.16.3.0/30 | HQ to B2 = 172.16.3.4/30.
• IP Rules: Routers get the 1st host address (.1 or network+1); PCs get the .10 address in their subnet.

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | S0/0/0 (DCE) | 172.16.3.1 | 255.255.255.252 | N/A |
| HQ | S0/0/1 (DCE) | 172.16.3.5 | 255.255.255.252 | N/A |
| HQ | Fa0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
| HQ | Fa0/1 | 172.16.2.1 | 255.255.255.0 | N/A |
| B1 | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
| B1 | Fa0/0 | 172.16.16.1 | 255.255.255.0 | N/A |
| B1 | Fa0/1 | 172.16.17.1 | 255.255.255.0 | N/A |
| B1 | Fa1/0 | 172.16.18.1 | 255.255.255.0 | N/A |
| B1 | Fa1/1 | 172.16.19.1 | 255.255.255.0 | N/A |
| B2 | S0/0/0 | 172.16.3.6 | 255.255.255.252 | N/A |
| B2 | Fa0/0 | 172.16.32.1 | 255.255.255.0 | N/A |
| B2 | Fa0/1 | 172.16.33.1 | 255.255.255.0 | N/A |
| B2 | Fa1/0 | 172.16.34.1 | 255.255.255.0 | N/A |
| B2 | Fa1/1 | 172.16.35.1 | 255.255.255.0 | N/A |
| PC1 | NIC (HQ LAN 1) | 172.16.1.10 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC (HQ LAN 2) | 172.16.2.10 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC (B1 LAN 1) | 172.16.16.10 | 255.255.255.0 | 172.16.16.1 |
| PC4 | NIC (B1 LAN 2) | 172.16.17.10 | 255.255.255.0 | 172.16.17.1 |
| PC5 | NIC (B1 LAN 3) | 172.16.18.10 | 255.255.255.0 | 172.16.18.1 |
| PC6 | NIC (B1 LAN 4) | 172.16.19.10 | 255.255.255.0 | 172.16.19.1 |
| PC7 | NIC (B2 LAN 1) | 172.16.32.10 | 255.255.255.0 | 172.16.32.1 |
| PC8 | NIC (B2 LAN 2) | 172.16.33.10 | 255.255.255.0 | 172.16.33.1 |
| PC9 | NIC (B2 LAN 3) | 172.16.34.10 | 255.255.255.0 | 172.16.34.1 |

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | S0/0/0 (DCE) | 172.16.3.1 | 255.255.255.252 | N/A |
| HQ | S0/0/1 (DCE) | 172.16.3.5 | 255.255.255.252 | N/A |
| HQ | Fa0/0 | 172.16.1.1 | 255.255.255.0 | N/A |
| HQ | Fa0/1 | 172.16.2.1 | 255.255.255.0 | N/A |
| B1 | S0/0/0 | 172.16.3.2 | 255.255.255.252 | N/A |
| B1 | Fa0/0 | 172.16.16.1 | 255.255.255.0 | N/A |
| B1 | Fa0/1 | 172.16.17.1 | 255.255.255.0 | N/A |
| B1 | Fa1/0 | 172.16.18.1 | 255.255.255.0 | N/A |
| B1 | Fa1/1 | 172.16.19.1 | 255.255.255.0 | N/A |
| B2 | S0/0/0 | 172.16.3.6 | 255.255.255.252 | N/A |
| B2 | Fa0/0 | 172.16.32.1 | 255.255.255.0 | N/A |
| B2 | Fa0/1 | 172.16.33.1 | 255.255.255.0 | N/A |
| B2 | Fa1/0 | 172.16.34.1 | 255.255.255.0 | N/A |
| B2 | Fa1/1 | 172.16.35.1 | 255.255.255.0 | N/A |
| PC1 | NIC (HQ LAN 1) | 172.16.1.10 | 255.255.255.0 | 172.16.1.1 |
| PC2 | NIC (HQ LAN 2) | 172.16.2.10 | 255.255.255.0 | 172.16.2.1 |
| PC3 | NIC (B1 LAN 1) | 172.16.16.10 | 255.255.255.0 | 172.16.16.1 |
| PC4 | NIC (B1 LAN 2) | 172.16.17.10 | 255.255.255.0 | 172.16.17.1 |
| PC5 | NIC (B1 LAN 3) | 172.16.18.10 | 255.255.255.0 | 172.16.18.1 |
| PC6 | NIC (B1 LAN 4) | 172.16.19.10 | 255.255.255.0 | 172.16.19.1 |
| PC7 | NIC (B2 LAN 1) | 172.16.32.10 | 255.255.255.0 | 172.16.32.1 |
| PC8 | NIC (B2 LAN 2) | 172.16.33.10 | 255.255.255.0 | 172.16.33.1 |
| PC9 | NIC (B2 LAN 3) | 172.16.34.10 | 255.255.255.0 | 172.16.34.1 |
| PC10 | NIC (B2 LAN 4) | 172.16.35.10 | 255.255.255.0 | 172.16.35.1 |