Packet Tracer 3.6.1 Skills Integration Challenge

Comprehensive Solution Guide: VLSM Address Calculation, Addressing Table, Static Route Summarization, and CLI Scripts

Task 1: VLSM Subnetting Calculations & Addressing Design

According to the challenge instructions, each router is allocated an independent base address space. Subnet zero is assigned to Fa0/0, subnet 1 to Fa0/1, subnet 2 to Fa1/0, and subnet 3 to Fa1/1. Router interfaces receive the first usable IP address in the subnet.

1. HQ Address Space: 192.168.0.128/25
• Host Requirement: 30 hosts per LAN (2 LANs: Fa0/0 and Fa0/1).
• Subnet Size: 30 hosts requires 5 host bits (2^5 - 2 = 30 hosts). Mask is /27 (255.255.255.224), block size = 32.
  - Subnet 0 (Fa0/0): 192.168.0.128/27 | Usable: 192.168.0.129 - 192.168.0.158 | Fa0/0 IP: 192.168.0.129
  - Subnet 1 (Fa0/1): 192.168.0.160/27 | Usable: 192.168.0.161 - 192.168.0.190 | Fa0/1 IP: 192.168.0.161

2. B1 Address Space: 192.168.1.0/24
• Host Requirement: 60 hosts per LAN (4 LANs: Fa0/0, Fa0/1, Fa1/0, Fa1/1).
• Subnet Size: 60 hosts requires 6 host bits (2^6 - 2 = 62 hosts). Mask is /26 (255.255.255.192), block size = 64.
  - Subnet 0 (Fa0/0): 192.168.1.0/26   | Usable: 192.168.1.1 - 192.168.1.62   | Fa0/0 IP: 192.168.1.1
  - Subnet 1 (Fa0/1): 192.168.1.64/26  | Usable: 192.168.1.65 - 192.168.1.126 | Fa0/1 IP: 192.168.1.65
  - Subnet 2 (Fa1/0): 192.168.1.128/26 | Usable: 192.168.1.129 - 192.168.1.190| Fa1/0 IP: 192.168.1.129
  - Subnet 3 (Fa1/1): 192.168.1.192/26 | Usable: 192.168.1.193 - 192.168.1.254| Fa1/1 IP: 192.168.1.193

3. B2 Address Space: 172.16.0.0/16
• Host Requirement: 1,000 hosts per LAN (4 LANs: Fa0/0, Fa0/1, Fa1/0, Fa1/1).
• Subnet Size: 1,000 hosts requires 10 host bits (2^10 - 2 = 1,022 hosts). Mask is /22 (255.255.252.0), block size = 4 in 3rd octet.
  - Subnet 0 (Fa0/0): 172.16.0.0/22 | Usable: 172.16.0.1 - 172.16.3.254 | Fa0/0 IP: 172.16.0.1
  - Subnet 1 (Fa0/1): 172.16.4.0/22 | Usable: 172.16.4.1 - 172.16.7.254 | Fa0/1 IP: 172.16.4.1
  - Subnet 2 (Fa1/0): 172.16.8.0/22 | Usable: 172.16.8.1 - 172.16.11.254| Fa1/0 IP: 172.16.8.1
  - Subnet 3 (Fa1/1): 172.16.12.0/22| Usable: 172.16.12.1 - 172.16.15.254| Fa1/1 IP: 172.16.12.1

4. B3 Address Space: 172.20.0.0/16
• Host Requirement: 5,000 hosts per LAN (4 LANs: Fa0/0, Fa0/1, Fa1/0, Fa1/1).
• Subnet Size: 5,000 hosts requires 13 host bits (2^13 - 2 = 8,190 hosts). Mask is /19 (255.255.224.0), block size = 32 in 3rd octet.
  - Subnet 0 (Fa0/0): 172.20.0.0/19  | Usable: 172.20.0.1 - 172.20.31.254  | Fa0/0 IP: 172.20.0.1
  - Subnet 1 (Fa0/1): 172.20.32.0/19 | Usable: 172.20.32.1 - 172.20.63.254 | Fa0/1 IP: 172.20.32.1
  - Subnet 2 (Fa1/0): 172.20.64.0/19 | Usable: 172.20.64.1 - 172.20.95.254 | Fa1/0 IP: 172.20.64.1
  - Subnet 3 (Fa1/1): 172.20.96.0/19 | Usable: 172.20.96.1 - 172.20.127.254| Fa1/1 IP: 172.20.96.1

Complete Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| HQ | Fa0/0 | 192.168.0.129 | 255.255.255.224 |
| HQ | Fa0/1 | 192.168.0.161 | 255.255.255.224 |
| HQ | S0/0/0 (DCE) | 10.0.0.1 | 255.255.255.252 |
| HQ | S0/0/1 (DCE) | 10.0.0.5 | 255.255.255.252 |
| HQ | S0/1/0 (DCE) | 10.0.0.9 | 255.255.255.252 |
| HQ | S0/1/1 | 209.165.201.2 | 255.255.255.252 |
| B1 | Fa0/0 | 192.168.1.1 | 255.255.255.192 |
| B1 | Fa0/1 | 192.168.1.65 | 255.255.255.192 |
| B1 | Fa1/0 | 192.168.1.129 | 255.255.255.192 |
| B1 | Fa1/1 | 192.168.1.193 | 255.255.255.192 |
| B1 | S0/0/0 | 10.0.0.2 | 255.255.255.252 |
| B2 | Fa0/0 | 172.16.0.1 | 255.255.252.0 |
| B2 | Fa0/1 | 172.16.4.1 | 255.255.252.0 |
| B2 | Fa1/0 | 172.16.8.1 | 255.255.252.0 |
| B2 | Fa1/1 | 172.16.12.1 | 255.255.252.0 |
| B2 | S0/0/0 | 10.0.0.6 | 255.255.255.252 |
| B3 | Fa0/0 | 172.20.0.1 | 255.255.224.0 |
| B3 | Fa0/1 | 172.20.32.1 | 255.255.224.0 |
| B3 | Fa1/0 | 172.20.64.1 | 255.255.224.0 |
| B3 | Fa1/1 | 172.20.96.1 | 255.255.224.0 |
| B3 | S0/0/0 | 10.0.0.10 | 255.255.255.252 |
| ISP | S0/0/0 (DCE) | 209.165.201.1 | 255.255.255.252 |
| ISP | Fa0/0 | 209.165.200.225 | 255.255.255.252 |
| Web Server | NIC | 209.165.200.226 | 255.255.255.252 (Gateway: 209.165.200.225) |

Task 3: Static & Default Routing Design (Using Exit Interfaces)

1. HQ Routing (3 Static Summary Routes + 1 Default Route out local exit interfaces):
• To B1 LANs (192.168.1.0/24 space): ip route 192.168.1.0 255.255.255.0 Serial0/0/0
• To B2 LANs (172.16.0.0 to 172.16.15.255 -> /20 summary): ip route 172.16.0.0 255.255.240.0 Serial0/0/1
• To B3 LANs (172.20.0.0 to 172.20.127.255 -> /17 summary): ip route 172.20.0.0 255.255.128.0 Serial0/1/0
• Default Route to ISP: ip route 0.0.0.0 0.0.0.0 Serial0/1/1

2. Branch Routers (B1, B2, B3) Routing (1 Default Route each out local exit interface):
• B1: ip route 0.0.0.0 0.0.0.0 Serial0/0/0
• B2: ip route 0.0.0.0 0.0.0.0 Serial0/0/0
• B3: ip route 0.0.0.0 0.0.0.0 Serial0/0/0

3. ISP Routing (Exactly 7 Static Routes pointing to HQ out Serial0/0/0):
• HQ LANs summary: ip route 192.168.0.128 255.255.255.128 Serial0/0/0
• B1 LANs summary: ip route 192.168.1.0 255.255.255.0 Serial0/0/0
• B2 LANs summary: ip route 172.16.0.0 255.255.240.0 Serial0/0/0
• B3 LANs summary: ip route 172.20.0.0 255.255.128.0 Serial0/0/0
• WAN Link HQ-B1: ip route 10.0.0.0 255.255.255.252 Serial0/0/0
• WAN Link HQ-B2: ip route 10.0.0.4 255.255.255.252 Serial0/0/0
• WAN Link HQ-B3: ip route 10.0.0.8 255.255.255.252 Serial0/0/0

Task 2 & 3: Complete Router CLI Configuration Scripts

1. Router HQ Configuration Script:

Router> enable Router# configure terminal Router(config)# hostname HQ HQ(config)# enable secret class HQ(config)# line console 0 HQ(config-line)# password cisco HQ(config-line)# login HQ(config-line)# exit HQ(config)# line vty 0 4 HQ(config-line)# password cisco HQ(config-line)# login HQ(config-line)# exit HQ(config)# interface FastEthernet 0/0 HQ(config-if)# ip address 192.168.0.129 255.255.255.224 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface FastEthernet 0/1 HQ(config-if)# ip address 192.168.0.161 255.255.255.224 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/0/0 HQ(config-if)# ip address 10.0.0.1 255.255.255.252 HQ(config-if)# clock rate 64000 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/0/1 HQ(config-if)# ip address 10.0.0.5 255.255.255.252 HQ(config-if)# clock rate 64000 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/1/0 HQ(config-if)# ip address 10.0.0.9 255.255.255.252 HQ(config-if)# clock rate 64000 HQ(config-if)# no shutdown HQ(config-if)# exit HQ(config)# interface Serial 0/1/1 HQ(config-if)# ip address 209.165.201.2 255.255.255.252 HQ(config-if)# no shutdown HQ(config-if)# exit ! 3 Static Routes + 1 Default Route HQ(config)# ip route 192.168.1.0 255.255.255.0 Serial0/0/0 HQ(config)# ip route 172.16.0.0 255.255.240.0 Serial0/0/1 HQ(config)# ip route 172.20.0.0 255.255.128.0 Serial0/1/0 HQ(config)# ip route 0.0.0.0 0.0.0.0 Serial0/1/1 HQ(config)# end HQ# copy running-config startup-config

2. Router B1 Configuration Script:

Router> enable Router# configure terminal Router(config)# hostname B1 B1(config)# enable secret class B1(config)# line console 0 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# line vty 0 4 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# interface FastEthernet 0/0 B1(config-if)# ip address 192.168.1.1 255.255.255.192 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 0/1 B1(config-if)# ip address 192.168.1.65 255.255.255.192 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 1/0 B1(config-if)# ip address 192.168.1.129 255.255.255.192 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 1/1 B1(config-if)# ip address 192.168.1.193 255.255.255.192 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface Serial 0/0/0 B1(config-if)# ip address 10.0.0.2 255.255.255.252 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0/0 B1(config)# end B1# copy running-config startup-config

3. Router B2 Configuration Script:

Router> enable Router# configure terminal Router(config)# hostname B2 B2(config)# enable secret class B2(config)# line console 0 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# line vty 0 4 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# interface FastEthernet 0/0 B2(config-if)# ip address 172.16.0.1 255.255.252.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 0/1 B2(config-if)# ip address 172.16.4.1 255.255.252.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 1/0 B2(config-if)# ip address 172.16.8.1 255.255.252.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 1/1 B2(config-if)# ip address 172.16.12.1 255.255.252.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface Serial 0/0/0 B2(config-if)# ip address 10.0.0.6 255.255.255.252 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0/0 B2(config)# end B2# copy running-config startup-config

4. Router B3 Configuration Script:

Router> enable Router# configure terminal Router(config)# hostname B3 B3(config)# enable secret class B3(config)# line console 0 B3(config-line)# password cisco B3(config-line)# login B3(config-line)# exit B3(config)# line vty 0 4 B3(config-line)# password cisco B3(config-line)# login B3(config-line)# exit B3(config)# interface FastEthernet 0/0 B3(config-if)# ip address 172.20.0.1 255.255.224.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 0/1 B3(config-if)# ip address 172.20.32.1 255.255.224.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 1/0 B3(config-if)# ip address 172.20.64.1 255.255.224.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 1/1 B3(config-if)# ip address 172.20.96.1 255.255.224.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface Serial 0/0/0 B3(config-if)# ip address 10.0.0.10 255.255.255.252 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0/0 B3(config)# end B3# copy running-config startup-config

5. Router ISP Configuration Script:

Router> enable Router# configure terminal Router(config)# hostname ISP ISP(config)# enable secret class ISP(config)# line console 0 ISP(config-line)# password cisco ISP(config-line)# login ISP(config-line)# exit ISP(config)# line vty 0 4 ISP(config-line)# password cisco ISP(config-line)# login ISP(config-line)# exit ISP(config)# interface FastEthernet 0/0 ISP(config-if)# ip address 209.165.200.225 255.255.255.252 ISP(config-if)# no shutdown ISP(config-if)# exit ISP(config)# interface Serial 0/0/0 ISP(config-if)# ip address 209.165.201.1 255.255.255.252 ISP(config-if)# clock rate 64000 ISP(config-if)# no shutdown ISP(config-if)# exit ! Exactly 7 Static Routes pointing out local exit interface Serial 0/0/0 to HQ ISP(config)# ip route 192.168.0.128 255.255.255.128 Serial0/0/0 ISP(config)# ip route 192.168.1.0 255.255.255.0 Serial0/0/0 ISP(config)# ip route 172.16.0.0 255.255.240.0 Serial0/0/0 ISP(config)# ip route 172.20.0.0 255.255.128.0 Serial0/0/0 ISP(config)# ip route 10.0.0.0 255.255.255.252 Serial0/0/0 ISP(config)# ip route 10.0.0.4 255.255.255.252 Serial0/0/0 ISP(config)# ip route 10.0.0.8 255.255.255.252 Serial0/0/0 ISP(config)# end ISP# copy running-config startup-config

Task 5: Verification & Extended Ping Testing

1. Verification Commands: Execute 'show ip interface brief' and 'show ip route' on each router to confirm all interfaces are up/up and the routing table includes the configured summary and default routes.
2. Extended Ping: On each branch router, test LAN reachability to Web Server (209.165.200.226) using source IP addresses corresponding to each Fa interface to confirm 100% full mesh reachability.