Course Code: CCE-332,

Course Title: Peripheral and Interfacing Sessional

Group: 09

Comparison with related works:

| Name | Primary Function(s) | Locomotion & Control | Key Features & Innovation |
| --- | --- | --- | --- |
| TrashBot (Our Project) | Autonomous waste collection in public areas and smart segregation of metallic vs. non-metallic waste | 4-wheel differential drive platform controlled by a Raspberry Pi 3 Model B, utilizing HC-SR04 ultrasonic sensors for obstacle avoidance and an ESP-32 for IoT connectivity. | Features dual-mode operation. Uses a multi-degree-of-freedom robotic arm with MG995 servos to grab waste, and an inductive proximity sensor for automated segregation. Developed as a highly capable, low-cost prototype |
| The V Robot [1] | Indoor mixed waste collection and real-time autonomous sorting. | Wheeled, Raspberry Pi 4, webcam. | Uses OpenCV for real-time visual waste identification and a 3-joint servo arm for physical grasping. Demonstrates that practical segregation can be achieved on affordable hardware. |
| DustCart [2] | Door-to-door separate waste collection on demand in urban pedestrian zones. | Wheeled balancing robot, GPS navigation combined with pre-loaded maps, LiDAR/ultrasonic. | Operates on an on-demand basis where users summon it via phone call or SMS. Features a touchscreen interface for users to deposit waste, and simultaneously monitors urban air quality using embedded sensors. |
| Trombia Free [3] | High-performance, large-area outdoor street and industrial sweeping. | Fully autonomous, all-electric, 3D LiDAR, GNSS satellite positioning. | Uses a filter less cyclone dust control system and an automated emptying station for continuous, heavy-duty fine dust removal. |
| SmartBin Rover [4] | Mobile dustbin that patrols predefined routes and approaches users for waste disposal. | Wheeled, GPS, ultrasonic sensors, AI person-tracking (YOLO). | It responds to approaching people automatically to encourage proper waste disposal and reduce public littering. |

References:

[1] "Waste collection in busy indoor spaces using everyday hardware," International Journal of Creative Research Thoughts (IJCRT), Vol. 14, Issue 6, 2026.

[2] Ferri, G., et al., "DustCart, a Mobile Robot for Urban Environments: Experiments of Pollution Monitoring and Mapping during Autonomous Navigation in Urban Scenarios," Proceedings of the International Conference on Robotics and Automation (ICRA), 2010.

[3] Trombia Technologies, "Trombia Free - Autonomous Street Sweeper," Trombia.com, 2026.

[4] "Review paper on AI-powered autonomous moving dustbin using person tracking and object motion prediction," RJ Wave, 2025.