Packet Tracer 4.7.1 Skills Integration Challenge

End-to-End Solution Manual: Hierarchical Subnetting, Addressing Table, Static Route Summarization & Full CLI Scripts

Task 1: Subnetting Design & Address Allocations

The network design divides two primary address spaces: WAN connections using 10.0.1.0/28, and Branch LANs using 10.1.0.0/16.

1. WAN Subnetting (10.0.1.0/28 Space):
• Base: 10.0.1.0/28. To provide four point-to-point /30 links (2 hosts each), borrow 2 additional bits: 28 + 2 = /30 (255.255.255.252). Block size is 4.
• Subnet 0 (R1 <-> B1): 10.0.1.0/30 | Usable: 10.0.1.1 (R1 S0/0) and 10.0.1.2 (B1 S0/0)
• Subnet 1 (R1 <-> B2): 10.0.1.4/30 | Usable: 10.0.1.5 (R1 S0/1) and 10.0.1.6 (B2 S0/0)
• Subnet 2 (R1 <-> B3): 10.0.1.8/30 | Usable: 10.0.1.9 (R1 S0/2) and 10.0.1.10 (B3 S0/0)
• Subnet 3 (R1 <-> B4): 10.0.1.12/30 | Usable: 10.0.1.13 (R1 S0/3) and 10.0.1.14 (B4 S0/0)

2. LAN Subnetting (10.1.0.0/16 Space):
• Step A: Divide 10.1.0.0/16 into four equal subnets for the four branch routers:
  - Borrow 2 bits: 16 + 2 = /18 (255.255.192.0). Block size in 2nd octet is 64.
  - Subnet 0 (B1 LAN space): 10.1.0.0/18
  - Subnet 1 (B2 LAN space): 10.1.64.0/18
  - Subnet 2 (B3 LAN space): 10.1.128.0/18
  - Subnet 3 (B4 LAN space): 10.1.192.0/18

• Step B: Divide each /18 branch space into four equal LAN subnets:
  - Borrow 2 bits: 18 + 2 = /20 (255.255.240.0). Block size in 2nd octet is 16.
  - B1 LANs: Fa0/0: 10.1.0.0/20 | Fa0/1: 10.1.16.0/20 | Fa1/0: 10.1.32.0/20 | Fa1/1: 10.1.48.0/20
  - B2 LANs: Fa0/0: 10.1.64.0/20 | Fa0/1: 10.1.80.0/20 | Fa1/0: 10.1.96.0/20 | Fa1/1: 10.1.112.0/20
  - B3 LANs: Fa0/0: 10.1.128.0/20 | Fa0/1: 10.1.144.0/20 | Fa1/0: 10.1.160.0/20 | Fa1/1: 10.1.176.0/20
  - B4 LANs: Fa0/0: 10.1.192.0/20 | Fa0/1: 10.1.208.0/20 | Fa1/0: 10.1.224.0/20 | Fa1/1: 10.1.240.0/20
  - First IP of each subnet is assigned to the corresponding router interface.

Complete Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | S0/0 (DCE) | 10.0.1.1 | 255.255.255.252 |
| R1 | S0/1 (DCE) | 10.0.1.5 | 255.255.255.252 |
| R1 | S0/2 (DCE) | 10.0.1.9 | 255.255.255.252 |
| R1 | S0/3 (DCE) | 10.0.1.13 | 255.255.255.252 |
| R1 | S1/0 | 209.165.201.2 | 255.255.255.252 |
| B1 | Fa0/0 | 10.1.0.1 | 255.255.240.0 |
| B1 | Fa0/1 | 10.1.16.1 | 255.255.240.0 |
| B1 | Fa1/0 | 10.1.32.1 | 255.255.240.0 |
| B1 | Fa1/1 | 10.1.48.1 | 255.255.240.0 |
| B1 | S0/0 | 10.0.1.2 | 255.255.255.252 |
| B2 | Fa0/0 | 10.1.64.1 | 255.255.240.0 |
| B2 | Fa0/1 | 10.1.80.1 | 255.255.240.0 |
| B2 | Fa1/0 | 10.1.96.1 | 255.255.240.0 |
| B2 | Fa1/1 | 10.1.112.1 | 255.255.240.0 |
| B2 | S0/0 | 10.0.1.6 | 255.255.255.252 |
| B3 | Fa0/0 | 10.1.128.1 | 255.255.240.0 |
| B3 | Fa0/1 | 10.1.144.1 | 255.255.240.0 |
| B3 | Fa1/0 | 10.1.160.1 | 255.255.240.0 |
| B3 | Fa1/1 | 10.1.176.1 | 255.255.240.0 |
| B3 | S0/0 | 10.0.1.10 | 255.255.255.252 |
| B4 | Fa0/0 | 10.1.192.1 | 255.255.240.0 |
| B4 | Fa0/1 | 10.1.208.1 | 255.255.240.0 |
| B4 | Fa1/0 | 10.1.224.1 | 255.255.240.0 |
| B4 | Fa1/1 | 10.1.240.1 | 255.255.240.0 |
| B4 | S0/0 | 10.0.1.14 | 255.255.255.252 |
| ISP | S0/0 (DCE) | 209.165.201.1 | 255.255.255.252 |
| ISP | Fa0/0 | 209.165.200.225 | 255.255.255.252 |
| Web Server | NIC | 209.165.200.226 | 255.255.255.252 |

Task 3: Static & Default Route Design (Using Exit Interface)

1. Router R1 Routing (4 Static Summary Routes + 1 Default Route):
• To B1 LANs: ip route 10.1.0.0 255.255.192.0 Serial0/0
• To B2 LANs: ip route 10.1.64.0 255.255.192.0 Serial0/1
• To B3 LANs: ip route 10.1.128.0 255.255.192.0 Serial0/2
• To B4 LANs: ip route 10.1.192.0 255.255.192.0 Serial0/3
• Default Route to ISP: ip route 0.0.0.0 0.0.0.0 Serial1/0

2. Branch Routers B1, B2, B3, B4 Routing (1 Default Route each):
• B1: ip route 0.0.0.0 0.0.0.0 Serial0/0
• B2: ip route 0.0.0.0 0.0.0.0 Serial0/0
• B3: ip route 0.0.0.0 0.0.0.0 Serial0/0
• B4: ip route 0.0.0.0 0.0.0.0 Serial0/0

3. ISP Router Routing (Exactly 2 Static Routes pointing to R1):
• Static Route for LAN address space: ip route 10.1.0.0 255.255.0.0 Serial0/0
• Static Route for WAN address space: ip route 10.0.1.0 255.255.255.240 Serial0/0

Task 2 & 3: Complete Ready-to-Paste CLI Configuration Scripts

1. Router R1 Configuration:

Router> enable Router# configure terminal Router(config)# hostname R1 R1(config)# enable secret class R1(config)# line console 0 R1(config-line)# password cisco R1(config-line)# login R1(config-line)# exit R1(config)# line vty 0 4 R1(config-line)# password cisco R1(config-line)# login R1(config-line)# exit ! WAN Links to Branch Routers (DCE with clock rate 64000) R1(config)# interface Serial 0/0 R1(config-if)# ip address 10.0.1.1 255.255.255.252 R1(config-if)# clock rate 64000 R1(config-if)# no shutdown R1(config-if)# exit R1(config)# interface Serial 0/1 R1(config-if)# ip address 10.0.1.5 255.255.255.252 R1(config-if)# clock rate 64000 R1(config-if)# no shutdown R1(config-if)# exit R1(config)# interface Serial 0/2 R1(config-if)# ip address 10.0.1.9 255.255.255.252 R1(config-if)# clock rate 64000 R1(config-if)# no shutdown R1(config-if)# exit R1(config)# interface Serial 0/3 R1(config-if)# ip address 10.0.1.13 255.255.255.252 R1(config-if)# clock rate 64000 R1(config-if)# no shutdown R1(config-if)# exit ! WAN Link to ISP R1(config)# interface Serial 1/0 R1(config-if)# ip address 209.165.201.2 255.255.255.252 R1(config-if)# no shutdown R1(config-if)# exit ! 4 Static Routes + 1 Default Route R1(config)# ip route 10.1.0.0 255.255.192.0 Serial0/0 R1(config)# ip route 10.1.64.0 255.255.192.0 Serial0/1 R1(config)# ip route 10.1.128.0 255.255.192.0 Serial0/2 R1(config)# ip route 10.1.192.0 255.255.192.0 Serial0/3 R1(config)# ip route 0.0.0.0 0.0.0.0 Serial1/0 R1(config)# end R1# copy running-config startup-config

2. Router B1 Configuration:

Router> enable Router# configure terminal Router(config)# hostname B1 B1(config)# enable secret class B1(config)# line console 0 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# line vty 0 4 B1(config-line)# password cisco B1(config-line)# login B1(config-line)# exit B1(config)# interface FastEthernet 0/0 B1(config-if)# ip address 10.1.0.1 255.255.240.0 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 0/1 B1(config-if)# ip address 10.1.16.1 255.255.240.0 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 1/0 B1(config-if)# ip address 10.1.32.1 255.255.240.0 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface FastEthernet 1/1 B1(config-if)# ip address 10.1.48.1 255.255.240.0 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# interface Serial 0/0 B1(config-if)# ip address 10.0.1.2 255.255.255.252 B1(config-if)# no shutdown B1(config-if)# exit B1(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0 B1(config)# end B1# copy running-config startup-config

3. Router B2 Configuration:

Router> enable Router# configure terminal Router(config)# hostname B2 B2(config)# enable secret class B2(config)# line console 0 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# line vty 0 4 B2(config-line)# password cisco B2(config-line)# login B2(config-line)# exit B2(config)# interface FastEthernet 0/0 B2(config-if)# ip address 10.1.64.1 255.255.240.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 0/1 B2(config-if)# ip address 10.1.80.1 255.255.240.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 1/0 B2(config-if)# ip address 10.1.96.1 255.255.240.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface FastEthernet 1/1 B2(config-if)# ip address 10.1.112.1 255.255.240.0 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# interface Serial 0/0 B2(config-if)# ip address 10.0.1.6 255.255.255.252 B2(config-if)# no shutdown B2(config-if)# exit B2(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0 B2(config)# end B2# copy running-config startup-config

4. Router B3 Configuration:

Router> enable Router# configure terminal Router(config)# hostname B3 B3(config)# enable secret class B3(config)# line console 0 B3(config-line)# password cisco B3(config-line)# login B3(config-line)# exit B3(config)# line vty 0 4 B3(config-line)# password cisco B3(config-line)# login B3(config-line)# exit B3(config)# interface FastEthernet 0/0 B3(config-if)# ip address 10.1.128.1 255.255.240.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 0/1 B3(config-if)# ip address 10.1.144.1 255.255.240.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 1/0 B3(config-if)# ip address 10.1.160.1 255.255.240.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface FastEthernet 1/1 B3(config-if)# ip address 10.1.176.1 255.255.240.0 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# interface Serial 0/0 B3(config-if)# ip address 10.0.1.10 255.255.255.252 B3(config-if)# no shutdown B3(config-if)# exit B3(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0 B3(config)# end B3# copy running-config startup-config

5. Router B4 Configuration:

Router> enable Router# configure terminal Router(config)# hostname B4 B4(config)# enable secret class B4(config)# line console 0 B4(config-line)# password cisco B4(config-line)# login B4(config-line)# exit B4(config)# line vty 0 4 B4(config-line)# password cisco B4(config-line)# login B4(config-line)# exit B4(config)# interface FastEthernet 0/0 B4(config-if)# ip address 10.1.192.1 255.255.240.0 B4(config-if)# no shutdown B4(config-if)# exit B4(config)# interface FastEthernet 0/1 B4(config-if)# ip address 10.1.208.1 255.255.240.0 B4(config-if)# no shutdown B4(config-if)# exit B4(config)# interface FastEthernet 1/0 B4(config-if)# ip address 10.1.224.1 255.255.240.0 B4(config-if)# no shutdown B4(config-if)# exit B4(config)# interface FastEthernet 1/1 B4(config-if)# ip address 10.1.240.1 255.255.240.0 B4(config-if)# no shutdown B4(config-if)# exit B4(config)# interface Serial 0/0 B4(config-if)# ip address 10.0.1.14 255.255.255.252 B4(config-if)# no shutdown B4(config-if)# exit B4(config)# ip route 0.0.0.0 0.0.0.0 Serial0/0 B4(config)# end B4# copy running-config startup-config

6. Router ISP Configuration:

Router> enable Router# configure terminal Router(config)# hostname ISP ISP(config)# enable secret class ISP(config)# line console 0 ISP(config-line)# password cisco ISP(config-line)# login ISP(config-line)# exit ISP(config)# line vty 0 4 ISP(config-line)# password cisco ISP(config-line)# login ISP(config-line)# exit ISP(config)# interface FastEthernet 0/0 ISP(config-if)# ip address 209.165.200.225 255.255.255.252 ISP(config-if)# no shutdown ISP(config-if)# exit ISP(config)# interface Serial 0/0 ISP(config-if)# ip address 209.165.201.1 255.255.255.252 ISP(config-if)# clock rate 64000 ISP(config-if)# no shutdown ISP(config-if)# exit ! Exactly 2 Static Routes pointing out Serial 0/0 to R1 ISP(config)# ip route 10.1.0.0 255.255.0.0 Serial0/0 ISP(config)# ip route 10.0.1.0 255.255.255.240 Serial0/0 ISP(config)# end ISP# copy running-config startup-config

Task 4: Connectivity & Verification Testing

1. Routing Table Verification: Run 'show ip route' on R1, the branch routers, and ISP to ensure all static and default routes are active.
2. Interface Status: Run 'show ip interface brief' to ensure all configured LAN and WAN interfaces report 'Status: up, Protocol: up'.
3. Extended Ping Verification: On each branch router, verify reachability to the Web Server (209.165.200.226) using an extended ping sourced from its Fa0/0 interface (e.g., 'ping 209.165.200.226 source 10.1.0.1' on B1).